| Supplementary Table 1. Summary of magnetic resonance imaging parameters. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Ax T1WI IDEAL | Ax T2WI IDEAL | Ax rFOV DWI | IDEAL IQ, B0 measurement | Cube CEST, Phase Cycle RF | Ax T1WI IDEAL +C |
| Pulse Sequence | 2D FSE-XL | 2D FSE-XL | 2D Spin Echo | 3D IDEAL IQ | 3D CUBE | 2D FSE-XL |
| Repetition time (ms) | 724.0 | 5317.0 | 2586.0 | 6.3 | 3000.0 | 724.0 |
| Echo time (ms) | 12.3 | 68.0 | 68.9 | 2.6 | 15.0 | 9.2 |
| Flip angle (degree) | 111 | 111 | N/A | 3 | N/A | 111 |
| Number of Echoes | 1 | 1 | 1 | 6 | 1 | 1 |
| Echo Train Length | 4 | 16 | N/A | 2 | 70 | 4 |
| Field of view(cm) | 24.0×24.0 | 24.0×24.0 | 18.0×9.0 | 20.0×15.0 | 20.0×15.0 | 24.0×24.0 |
| Slice thickness (mm) | 4.0 | 4.0 | 4.0 | 8.0 | 8.0 | 4.0 |
| Slice Spacing (mm) | 1.0 | 1.0 | 0 | 0 | 0 | 1.0 |
| Number of slices | 36 | 36 | 10 | 10 | 10 | 36 |
| b factors (s/mm2) | N/A | N/A | 0, 50, 800 | N/A | N/A | N/A |
| Matrix | 320×256 | 320×256 | 128×96 | 128×96 | 128×96 | 320×256 |
| NEX | 2 | 1 | 1, 10 | 1 | 1 | 2 |
| Total scan time (min:sec) | 1 min 53sec | 1 min 52sec | 1 min 17sec | 21sec | 4 min 56sec | 3 min 55sec |
| Note: CEST: Chemical exchange saturation transfer, IDEAL-IQ: Iterative Decomposition of water and fat with Echo Asymmetry and Least squares estimation, CUBE: three-dimensional fast spin-echo with an extended echo train acquisition. |  |  |  |  |  |  |