Supplementary Materials

Supplementary Data S1 Detailed parameters of CT scanning

Scanning parameters included:​​ tube voltage 120 kVp with automatic tube current modulation; detector collimation 128×0.625 mm or 40×0.625 mm; matrix size 512×512; slice thickness 3 mm; and reconstruction interval 1 mm. All patients underwent non-contrast abdominal CT followed by contrast-enhanced CT acquisitions in the arterial phase (AP) and portal venous phase (VP).

​	The contrast-enhanced CT protocol was as follows:​​ After a 6-hour fast, patients ingested 500–1000 mL of negative oral contrast for bowel opacification. Iodinated contrast medium (Ultravist 370; 1.5 ml/kg) was administered intravenously via an antecubital vein at 2.5–3.5 ml/s using a power injector. AP scanning commenced 35 seconds post-injection; VP scanning commenced at 70 seconds. Images were transferred to a Picture archiving and communication system workstation for analysis.

​Key Improvements and Rationale:​TABLE 1 Detailed parameters of CT scanning

| Parameters | Siemens SOMATOM Definition AS 128 | Siemens SOMATOM Definition AS 40 |
| --- | --- | --- |
| Tube voltage (kVp) | 120 | 120 |
| Tube current (mA) | Auto | Auto |
| Rotation times (s) | 0.5 | 0.5 |
| Field of view (mm) | 300-450 | 300-500 |
| Detector collimation (mm) | 128×0.625 | 40×0.625 |
| Pixel size | 512×512 | 512×512 |
| Slice thickness (mm) | 3 | 3 |

Supplementary Data S2. Derivation of Clinical Prediction Model

Risk Score = -1.2011088349836248 - 0.6910950488342797 × Edmondson+ 1.0336199661056356 × MVI + 1.142063401160561 × Ki67 + 0.8644773008089934 × HBsAg + 0.007059663906400884 × PT

Supplementary Data S3. Model performance of each cohort

| Model | Model | AUC (95% CI) | ACC | TPR | TNR | PPV | NPV | BER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training cohort | Clinics | 0.71 (0.624-0.795) | 0.634 | 0.617 | 0.656 | 0.704 | 0.563 | 0.363 |
|  | Habitat | 0.822 (0.755-0.889) | 0.683 | 0.519 | 0.902 | 0.875 | 0.585 | 0.290 |
|  | EX5MM | 0.777 (0.699-0.855) | 0.711 | 0.667 | 0.770 | 0.794 | 0.635 | 0.281 |
|  | EX8MM | 0.773 (0.694-0.852) | 0.683 | 0.580 | 0.820 | 0.810 | 0.595 | 0.300 |
|  | Clinics_Habitat_EX5MM_EX8MM | 0.912 (0.866-0.958) | 0.831 | 0.790 | 0.885 | 0.901 | 0.761 | 0.162 |
| Validation cohort | Clinics | 0.781 (0.661–0.902) | 0.689 | 0.632 | 0.783 | 0.828 | 0.562 | 0.293 |
|  | Habitat | 0.792 (0.681–0.903) | 0.656 | 0.553 | 0.826 | 0.840 | 0.528 | 0.311 |
|  | EX5MM | 0.747 (0.626–0.869) | 0.639 | 0.632 | 0.652 | 0.75 | 0.517 | 0.358 |
|  | EX8MM | 0.741 (0.618-0.865) | 0.689 | 0.553 | 0.913 | 0.913 | 0.553 | 0.267 |
|  | Clinics_Habitat_EX5MM_EX8MM | 0.927 (0.861-0.993) | 0.770 | 0.684 | 0.913 | 0.929 | 0.636 | 0.201 |

Supplementary Data S4. The results of the DeLong test in the training cohort.

| Model | Model | P | Z |
| --- | --- | --- | --- |
| Clinics_Habitat_EX5MM_EX8MM | Clinics | 0.00000181 | -4.77399 |
|  | EX5MM | 0.000109 | -3.87003 |
|  | EX8MM | 0.0000663 | -3.98915 |

Supplementary Data S5. The results of the DeLong test in the validation cohort.

| Model | Model | P | Z |
| --- | --- | --- | --- |
| Clinics_Habitat_EX5MM_EX8MM | Clinics | 0.010167 | -2.57008 |
|  | EX5MM | 0.002985 | -2.96926 |
|  | EX8MM | 0.00069 | -3.39336 |