| Additional file 1. MRI Scanning Protocol |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | TR9/TE10（ms） | Slice Thickness（mm） | Slice Gap（mm） | Matrix | Flip Angle | NEX11 |
| GE 1.5T/3.0T |  |  |  |  |  |  |
| T1WI（FSE1） | 622/10 |  |  |  | - |  |
| T2WI（STIR2） | 6330/68 | 5.0 | 0.5 | 384×224 | - | 1 |
| DWI（SS-EPI3/MUSE4） | 6330/64 | 5.0 | 0.5 | 128×128 | - | 1 |
| VIBRANT5（Sagittal） | 6.1/2.9 | 2.0 |  | 256×128 | 15° |  |
| VIBRANT（Delayed Axial） | 6.1/2.9 | 2.0 |  | 256×128 | 15° | 1 |
| Philips 3.0T |  |  |  |  |  |  |
| T1WI（TSE6） | 531/8 |  |  |  | - |  |
| T2WI（SPAIR7） | 4000/70 | 5.0 | 0.5 | 384×224 | - | 1 |
| DWI（SS-EPI） | 7475/49 | 5.0 | 0.5 | 128×128 | - | 1 |
| e-THRIVE8（Sagittal） | 4.4/2.2 | 2.0 |  | 256×128 | 12° |  |
| e-THRIVE（Delayed Axial） | 4.4/2.2 | 2.0 |  | 256×128 | 12° | 1 |
| Note：1 FSE=Fast spin echo；2 STIR=Short time of inversion recovery；3 SS-EPI=Single shot echo planar imaging；4 MUSE=Multiplexed sensitivity-encoding，；5 VIBRANT=Volume imaging for breast assessment；6 TSE=Turbo spin echo；7 SPAIR=Spectral attenuated inversion recovery；8 e-THRIVE=enhanced-T1 high resolution isotropic volume examination；9TR, repetition time； 10TE, echo time；11 NEX, number of excitations |  |  |  |  |  |  |

| Additional file 2. Clinicopathological Characteristics |  |  |  |
| --- | --- | --- | --- |
|  | Rapid Shrinkage | Slow Shrinkage | P-value |
| Age | 52.00（44.00,57.00） | 48.00（39.00,55.00） | 0.280a |
| Affected breast |  |  | 0.271 |
| left | 14（45.2） | 106（55.8） |  |
| right | 17（54.8） | 84（44.2） |  |
| Clinical T stage |  |  | 0.117 |
| early（Ⅰ-Ⅱ） | 10（32.3） | 90（47.4） |  |
| late（Ⅲ-Ⅳ） | 21（67.7） | 100（52.6） |  |
| ER |  |  | 0.027* |
| ＜1% | 17（54.8） | 65（34.2） |  |
| ≥1% | 14（45.2） | 125（65.8） |  |
| PR |  |  | 0.012* |
| ＜1% | 24（77.4） | 101（53.2） |  |
| ≥1% | 7（22.6） | 89（46.8） |  |
| HER2 |  |  | 0.006* |
| negative | 13（41.9） | 128（67.4） |  |
| positive | 18（58.1） | 62（32.6） |  |
| Ki-67 |  |  | 0.194 |
| ＜14% | 3（9.7） | 8（4.2） |  |
| ≥14% | 28（90.3） | 182（95.8） |  |
| Note: * indicates statistically significant difference. a: Wilcoxon test was used |  |  |  |

| Additional file 3. The Univariate Analysis of MRI Features Associated with Rapid Tumor shrinkage at Each Time Point |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline (Rapid/Slow) | P-value | 2nd Cycle (Rapid/Slow) | P-value | 4nd Cycle (Rapid/Slow) | P-value | 6nd Cycle (Rapid/Slow) | P-value |
| Lesion type |  | 0.058 |  | 0.294 |  | 0.002* |  | 0.003* |
| mass | 16（51.6）/131（68.9） |  | 13 ( 41.9)/99 (52.1) |  | 5（16.1）/87（45.8） |  | 4（12.9）/77（40.5） |  |
| non-mass | 15（48.4）/59（31.1） |  | 18 ( 58.1)/91 (47.9) |  | 26（83.9）/103（54.2） |  | 27（87.1）/113（59.5） |  |
| Multifocal/Multicentric |  | 0.817 |  | 0.71 |  | 0.314 |  | 0.352 |
| no | 28（90.3）/174（91.6） |  | 29（93.5）/174（91.6） |  | 30（96.8）/174（91.6） |  | 30（96.8）/175（92.1） |  |
| yes | 3（9.7）/16（8.4） |  | 2（6.5）/16（8.4） |  | 1（3.2）/16（8.4） |  | 1（3.2）/15（7.9） |  |
| Enhancement pattern |  | 0.663 |  | 0.731 |  | 0.115 |  | ＜0.001* |
| none | 0（0.0）/0（0.0） |  | 0（0.0）/0（0.0） |  | 1（3.2）/0（0.0） |  | 4（12.9）/0（0.0） |  |
| homogeneous | 0（0.0）/1（0.5） |  | 0（0.0）/3（1.6） |  | 0（0.0）/3（1.6） |  | 0（0.0）/3（1.6） |  |
| heterogeneous | 29（93.5）/164（86.3） |  | 29（93.5）/168（88.4） |  | 28（90.3）/176（92.6） |  | 25（80.6）/178（93.7） |  |
| rim | 2（6.5）/20（10.5） |  | 2（6.5）/15（7.9） |  | 2（6.5）/8（4.2） |  | 2（6.5）/6（3.2） |  |
| clustered ring | 0（0.0）/5（2.6） |  | 0（0.0）/4（2.1） |  | 0（0.0）/3（1.6） |  | 0（0.0）/3（1.6） |  |
| Peritumoral edema |  | 0.034* |  | 0.438 |  | 0.944 |  | 0.596 |
| no | 9（29.0）/94（49.5） |  | 18（58.1）/124（65.3） |  | 22（71.0）/136（71.6） |  | 25（80.6）/145（76.3） |  |
| yes | 22（71.0）/96（50.5） |  | 13（41.9）/66（34.7） |  | 9（29.0）/54（28.4） |  | 6（19.4）/45（23.7） |  |
| Necrosis |  | 0.383 |  | 0.357 |  | 0.25 |  | 0.395 |
| no | 27（87.1）/153（80.5） |  | 29（93.5）/167（87.9） |  | 30（96.8）/172（90.5） |  | 30（96.8）/176（92.6） |  |
| yes | 4（12.9）/37（19.5） |  | 2（6.5）/23（12.1） |  | 1（3.2）/18（9.5） |  | 1（3.2）/14（7.4） |  |
| Nipple |  | 0.185 |  | 0.056 |  | 0.023* |  | 0.026* |
| no | 26（83.9）/138（72.6） |  | 28（90.3）/142（74.7） |  | 29（93.5）/143（75.3） |  | 29（93.5）/144（75.8） |  |
| yes | 5（16.1）/52（27.4） |  | 3（9.7）/48（25.3） |  | 2（6.5）/47（24.7） |  | 2（6.5）/46（24.2） |  |
| Skin |  | 0.867 |  | 0.442 |  | 0.614 |  | 0.383 |
| no | 22（71.0）/132（69.5） |  | 25（80.6）/141（74.2） |  | 26（83.9）/152（80.0） |  | 27（87.1）/153（80.5） |  |
| yes | 9（29.0）/58（30.5） |  | 6（19.4）/49（25.8） |  | 5（16.1）/38（20.0） |  | 4（12.9）/37（19.5） |  |
| Chest wall |  | 0.361 |  | 0.361 |  | 0.481 |  | 0.415 |
| no | 31（100.0）/185（97.4） |  | 31（100.0）/185（97.4） |  | 31（100.0）/187（98.4） |  | 31（100.0）/186（97.9） |  |
| yes | 0（0.0）/5（2.6） |  | 0（0.0）/5（2.6） |  | 0（0.0）/3（1.6） |  | 0（0.0）/4（2.1） |  |
| Axillary lymph node |  | 0.3 |  | 0.109 |  | 0.213 |  | 0.253 |
| Normal | 0（0.0）/8（4.2） |  | 0（0.0）/8（4.2） |  | 0（0.0）/8（4.2） |  | 0（0.0）/9（4.7） |  |
| Cortical thickening | 9（29.0）/38（20.0） |  | 15（48.4）/59（31.1） |  | 17（54.8）/77（40.5） |  | 18（58.1）/86（45.3） |  |
| enlargement | 22（71.0）/144（75.8） |  | 16（51.6）/123（64.7） |  | 14（45.2）/105（55.3） |  | 13（41.9）/95（50.0） |  |
| Shrinkage pattern |  |  |  | 0.656 |  | 0.011* |  | 0.094 |
| CS |  |  | 17（54.8）/96（50.5） |  | 20（64.5）/76（40.0） |  | 14（45.2）/57（30.0） |  |
| non-CS |  |  | 14（45.2）/94（49.5） |  | 11（35.5）/114（60.0） |  | 17（54.8）/133（70.0） |  |
| T2WI |  | 0.646 |  | 0.516 |  | 0.043* |  | 0.017* |
| hypointense | 1（3.2）/10（5.3） |  | 0（0.0）/7（3.7） |  | 0（0.0）/9（4.7） |  | 0（0.0）/7（3.7） |  |
| isointense | 3（9.7）/28（14.7） |  | 7（22.6）/36（18.9） |  | 12（38.7）/38（20.0） |  | 14（45.2）/42（22.1） |  |
| hyperintense | 27（87.1）/152（80.0） |  | 24（77.4）/147（77.4） |  | 19（61.3）/143（75.3） |  | 17（54.8）/141（74.2） |  |
| DWI |  | 0.788 |  | 0.002* |  | ＜0.001* |  | ＜0.001* |
| hyperintense | 28（90.3）/178（93.7） |  | 23（74.2）/174（91.6） |  | 14（45.2）/165（86.8） |  | 14（45.2）/156（82.1） |  |
| rim hyperintense | 2（6.5）/8（4.2） |  | 2（6.5）/9（4.7） |  | 1（3.2）/7（3.7） |  | 1（3.2）/8（4.2） |  |
| isointense | 1（3.2）/4（2.1） |  | 6（19.4）/7（3.7） |  | 16（51.6）/18（9.5） |  | 16（51.6）/26（13.7） |  |
| TIC |  | 0.89 |  | 0.002* |  | 0.013* |  | 0.279 |
| persistent | 0（0.0）/1（0.5） |  | 21（67.7）/67（35.3） |  | 24（77.4）/98（51.6） |  | 24（77.4）/122（64.2） |  |
| plateau | 2（6.5）/10（5.3） |  | 4（12.9）/28（14.7） |  | 4（12.9）/25（13.2） |  | 3（9.7）/19（10.0） |  |
| washout | 29（93.5）/179（94.2） |  | 6（19.4）/95（50.0） |  | 3（9.7）/67（35.3） |  | 4（12.9）/49（25.8） |  |
| E1 lesion（%） | 190.80（163.55, 222.70）/202.35（175.25, 239.02） | 0.171 | 105.10（68.00，144.40）/157.75（112.22，197.95） | ＜0.001*a | 62.60（42.00，127.95）/127.65（75.48，174.33） | 0.001*a | 44.50（28.95，92.05）/118.55（69.67，162.65） | ＜0.001*a |
| E2 lesion（%） | 162.80（141.00, 182.75）/164.85（140.55, 187.60） | 0.704 | 142.70（110.05，186.75）/158.15（130.38，183.62） | 0.193a | 112.30（86.20，167.23） | 0.015*a | 94.20（65.25,121.75）/138.75（115.58,172.02） | ＜0.001*a |
| E1 gland（%） | 25.00（14.85, 36.90）/22.00（14.20, 34.30） | 0.602 | 15.70（9.85，26.15） | 0.254a | 13.60（9.75，20.00）/15.75（10.50，21.48） | 0.234a | 12.70（8.95，18.80）/14.30（10.30，21.15） | 0.286a |
| E2 gland（%） | 54.60（37.55, 71.65）/55.30（41.58, 76.65） | 0.736 | 39.20（28.90，58.50）/46.15（35.92，60.00） | 0.146a | 34.30（27.80，46.80）/41.50（32.33，53.75） | 0.054a | 31.50（27.35，48.25）/38.80（29.00，51.85） | 0.193a |
| Emax lesion（%） | 192.50（165.20, 222.70）/205.85 （180.90, 242.05） | 0.154 | 143.40（116.55，190.45）/180.00（148.55，207.60） | 0.007*a | 112.30（86.20，167.40）/154.75（118.78，193.48） | 0.002*a | 94.20（63.25，121.85）/155.35（121.53，194.60） | ＜0.001*a |
| SER1 | 8.68 （4.85, 12.01）/9.13 （5.94, 13.63） | 0.329 | 6.88（4.58，9.44）/8.48（5.63，12.67） | 0.068a | 5.60（3.25，8.70）/7.81（4.68，11.15） | 0.056a | 3.53（2.45，5.06）/7.17（4.56，10.92） | ＜0.001*a |
| SER2 | 3.10 （2.00, 3.85）/2.86 （2.12, 3.90） | 0.911 | 3.49（2.60，4.82）/3.35（2.42，4.20） | 0.613a | 3.04（2.33，4.20）/3.28（2.36，4.56） | 0.652a | 2.43（1.89，3.41）/3.54（2.81，4.71） | ＜0.001*a |
| ADClesion（×10-3mm2/s） | 1.01 （0.92, 1.11）/0.96 （0.87, 1.06） | 0.087 | 1.20（1.08，1.35）/1.12（1.00，1.28） | 0.027*a | 1.43（1.24，1.54）/1.21（1.06，1.41） | ＜0.001*a | 1.58（1.42，1.75）/1.30（1.09，1.47） | ＜0.001*a |
| ADCgland（×10-3mm2/s） | 1.59 （1.47, 1.75）/1.62 （1.47, 1.79） | 0.958 | 1.60（1.52，1.83）/1.64（1.51，1.79） | 0.825a | 1.65（1.55，1.83）/1.63（1.49，1.77） | 0.464a | 1.63（1.56，1.81）/1.62（1.50,1.80） | 0.279a |
| rADC | 0.61 （0.57, 0.70）/0.60 （0.51, 0.68） | 0.145 | 0.75（0.68，0.82）/0.70（0.61，0.80） | 0.036*a | 0.84（0.78，0.92）/0.77（0.65，0.87） | 0.003*a | 0.91（0.87，0.98）/0.79（0.68，0.90） | ＜0.001*a |
| Note: * indicates statistically significant difference. a: Wilcoxon test was used Abbreviations: CS concentric shrinkage; T2WI T2-weighted imaging; DWI diffusion-weighted imaging; TIC time–signal intensity curve; ADC apparent diffusion coefficient |  |  |  |  |  |  |  |  |

| Additional file 4. AUCs of Different Models and Pairwise Comparison Results |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AUC | P |  |  |  | 95%CI |
| Model 1 | 0.627 | Ref | 0.031* | 0.06 | ＜0.001* | 0.533-0.722 |
| Model 2 | 0.676 | 0.031* | Ref | 0.774 | 0.014* | 0.573-0.780 |
| Model 3 | 0.688 | 0.06 | 0.774 | Ref | 0.009* | 0.592-0.784 |
| Model 4 | 0.814 | ＜0.001* | 0.014* | 0.009* | Ref | 0.734-0.895 |
| Model 5 | 0.759 | 0.009* | 0.166 | 0.138 | 0.243 | 0.660-0.858 |
| Model 1：clinical-pathological features； Model 2：Clinicopathological and Baseline MRI Features；Model 3：Clinicopathological and Second-Cycle MRI Features；Model 4：Clinicopathological and Fourth-Cycle MRI Features；Model 5：Clinicopathological and Sixth-Cycle MRI Features Ref: reference group for comparison; *P < 0.05 indicates statistical significance |  |  |  |  |  |  |

| Additional file 5. Comparison of AUCs Between Combined Models Based on Clinicopathological, Baseline, and Cycle 4 vs. Cycle 6 MRI Features |  |  |  |
| --- | --- | --- | --- |
|  | AUC | P | 95%CI |
| Clinicopathological+Baseline+Cycle 4 MRI | 0.835 | Ref | 0.756-0.913 |
| Clinicopathological+Baseline+Cycle 6 MRI | 0.796 | 0.319 | 0.709-0.883 |
| Ref: reference group for comparison; *P < 0.05 indicates statistical significance |  |  |  |