Microstructure Mapping withTime-dependent Diffusion MRI Differentiates Primary Central Nervous System Lymphoma from Glioblastoma

ELECTRONIC SUPPLEMENTARY MATERIAL

Supplementary Text

Routine structural MRI

Routine structural MRI included T1-weighted images before and after Gd-enhancement, and T2-FLAIR images, with a total acquisition time of 17 min. The protocol parameters were as follows: CE-T1w (TE = 3.5 ms; TR = 7.9 ms; FOV = 250 × 199 × 170 mm3; slice thickness = 1 mm) and FLAIR (TE = 313 ms; TR = 4800 ms; FOV = 250 × 250 × 150 mm3; slice thickness = 2 mm).

Data Analysis and Principle of IMPULSED

td-MRI signals (S) were fitted using the IMPULSED model [1], a 2-compartment model comprising intra- (Sin) and extracellular (Sex) components:

S=Vin*Sin+(1-Vin)*Sex

where S is the measured td-MRI signal, Sin and Sex are the intracellular and extracellular signals, respectively, and Vinrepresents the volume fraction of the intracellular space. Sin uses sphere geometry and adapts for trapezoid OGSE waveforms [2]. Sex assumes a td-independent exponential decay with extracellular diffusivity (Dex).

Microstructural parameters including diameter (d), Vin, and Dex were estimated from Eq. 1 using a nonlinear least squares algorithm in Matlab R2024b (Mathworks, Natick, MA) with a fixed intracellular diffusion coefficient at 1.0 µm2/ms. The fitting was repeated 100 times with randomized initializations to avoid local minimums, under the following physiological constraints: 1< d <50 µm, 0.01< Vin <1, 0.5< d <3.5 µm2/ms. Cellularity was defined as  Vin / d * 100 [3]. ADC maps were calculated at each td according to the log-linear fitting  S/S0=e-bD to obtain ADC0Hz (PGSE), ADC25Hz and ADC50Hz. The change in ADC (rADC1, rADC2) were calculated using the combination of the ADC:

rADC1=(ADC25Hz-ADC0Hz)ADC25Hz *100

rADC2=ADC50Hz-ADC0HzADC50Hz *100

References

1.	Jiang X, Li H, Xie J, et al. In vivo imaging of cancer cell size and cellularity using temporal diffusion spectroscopy. Magn Reson Med 2017;78:156-164. https://doi.org/10.1002/mrm.26356.

2.	Xu J, Jiang X, Li H, et al. Magnetic resonance imaging of mean cell size in human breast tumors. Magn Reson Med 2020;83:2002-2014. https://doi.org/10.1002/mrm.28056.

3.	Wu D, Jiang K, Li H, et al. Time-Dependent Diffusion MRI for Quantitative Microstructural Mapping of Prostate Cancer. Radiology 2022;303:578-587. https://doi.org/10.1148/radiol.211180.

Supplementary Figures

Supplementary Figure S1. Ridge trace plot for penalty parameter (K) selection.

Supplementary Figure S2. (a-b) ADC values measured at 0 Hz, 25 Hz, 50 Hz in tumor region or (c-d) peritumoral edema in GBM and PCNSL. * p < 0.05, ** p < 0.01, *** p < 0.001，ns show no significance.

Supplementary Figure S3. Correlations between the ADC at 0 Hz and cellularity(a) or diameter(b) estimated from td-dMRI.

Supplementary Tables

Supplementary Table S1. Multicollinearity diagnostics using ordinary least squares (OLS) prior to ridge regression.

| Variable | VIF |
| --- | --- |
| Vin | 464.618 |
| Diameter | 49.976 |
| Cellularity | 224.536 |
| Dex | 79.701 |
| ADC0Hz | 345.994 |
| ADC25Hz | 686.226 |
| ADC50Hz | 37.868 |
| rADC1 | 86.380 |
| rADC2 | 29.788 |

Note: All VIF values are substantially above 10, indicating severe multicollinearity among predictors. This justifies the use of ridge regression (L2 penalization) to obtain stable coefficient estimates. VIF: variance inflation factor.

Supplementary Table S2.  Comparison of patient information between PCNSL and GBM.

|  | PCNSL (n = 19) | GBM (n = 32) | p value |
| --- | --- | --- | --- |

| Gender Male Female | 10 (52.6%) 9 (47.4%) | 20 (62.5%) 12 (37.5%) | 0.489 |  |  |
| --- | --- | --- | --- | --- | --- |
| Age | 59.95 ± 2.03 | 58.28 ± 2.20 | 0.785 |  |  |
| Number of lesions (%) Single Multiple | 12 (63.2%) 7 (36.8%) | 21 (65.6%) 11 (34.4%) |  | 0.859 |
| Location (%) Only supratentorial Only infratentorial Supra-and infratentorial | 15 (78.9%) 0 (0%) 4 (21.1%) | 31 (96.9%) 1 (3.1%) 0 (0%) |  | 0.016 |

Note: Continuous variables are expressed as mean ± standard deviation, while categorical variables are presented as number (percentage).

Supplementary Table S7. Results of binary logistic regression analysis for differentiating PCNSL from GBM.

| Variable | B | Wald χ2 | p value |
| --- | --- | --- | --- |
| Vin | 1.893 | 3.103 | 0.078 |
| Cellularity | 0.858 | 0.741 | 0.389 |
| Constant | -0.837 | 3.719 | 0.054 |

Note: Vin = intracellular fraction.