# Supplementary Table S1. Comparative studies on wearable EEG devices

Table S1 summarizes representative comparative studies evaluating consumer- and research-grade EEG devices. The table outlines the study year, devices compared, experimental setting, evaluation metrics, key outcomes, and the main methodological gaps highlighted by each work.

| Year | Authors | Devices compared | Setting / Task | Metrics used | Key outcomes | Identified gaps |
| --- | --- | --- | --- | --- | --- | --- |
| 2017 | Ratti et al. | Medical-grade (B-Alert, Enobio) vs Consumer (Muse, Mindwave) | Resting-state (eyes open/closed) | PSD, SNR, test–retest reliability | Consumer devices captured main EEG bands but showed higher variability and artefacts. | Controlled conditions only; no dynamic or ecological tasks. |
| 2022 | Gao et al. | Wearable frontal EEG (Mindeep) vs Neuroscan | Resting-state, auditory oddball, attention tasks | Cross-correlation, ERP amplitude, classification accuracy | Wearable showed high correlation with reference system but reduced ERP amplitude. | Frontal-only layout; limited spatial coverage; small sample size. |
| 2023 | Mahdid et al. | Research-grade vs four consumer-grade headsets | Functional connectivity analysis across tasks | Connectivity strength, coherence patterns | Wearables reproduced main connectivity trends but with weaker coherence and stability. | Few ecological tasks; limited multi-metric validation. |
| 2024 | Mikhaylov et al. | Research-grade vs three consumer-grade (Muse S, PSBD Headband, Lite Headphones) | Resting-state recordings | PSD, signal quality indices, electrode-wise comparisons | Consumer headsets showed increased low-frequency power and artefacts. | No task-based evaluation; only resting-state data. |
| 2025 | Arpaia et al. | Systematic review of 58 studies on wearable EEG | Various real-world and motion contexts | Artifact detection and correction techniques | Identified motion- and electrode-related artefacts as main limitations for wearable EEG. | Scarce analysis of neurometric validity and subjective correlation. |

# References

- Ratti, E., Waninger, S., Berka, C., Ruffini, G., Verma, A., & Menicucci, D. (2017). Comparison of medical and consumer wireless EEG systems for research use: A technical review. Sensors, 17(12), 2333. https://doi.org/10.3390/s17122333

- Gao, L., Chen, H., Zhou, C., Zhang, Y., & Li, L. (2022). Signal quality investigation of a new wearable frontal-lobe EEG device. Sensors, 22(5), 1898. https://doi.org/10.3390/s22051898

- Y. Mahdid, U. Lee and S. Blain-Moraes, "Assessing the Quality of Wearable EEG Systems Using Functional Connectivity," in IEEE Access, vol. 8, pp. 193214-193225, 2020, doi: 10.1109/ACCESS.202

- Mikhaylov, A., Averbukh, N., Kabanova, E., Kuznetsov, I., & Gaiduk, P. (2024). Comparison of EEG signal spectral characteristics obtained with consumer- and research-grade devices. Sensors, 24(24), 8108. https://doi.org/10.3390/s24248108

- Arpaia, P., Baccigalupi, A., & Daponte, P. (2025). A systematic review of techniques for artifact detection in wearable EEG. Sensors, 25(18), 5770. https://doi.org/10.3390/s25185770