cGMP-Compliant High-Yield Automated Production of [18F]AlF-FAPI-74: Optimization of Quality Control and Evaluation of Molar Dose Impact

Supplementary information

Ward Storms1, Sofie Celen2, Justine Maes2, Kim Serdons2, Karolien Goffin2, Tjibbe De Groot2, Laura Trump3, Corentin Warnier3, Thibeau Gendron4 Romaric Gerardy3, Koen Van Laere2, Christophe M. Deroose2, Janke Kleynhans1, and Frederik Cleeren1*

1Radiopharmaceutical Research, Department of Pharmaceutical and Pharmacological Sciences, KU Leuven, Belgium

2 Nuclear Medicine, University Hospital Leuven, Leuven, Belgium

3Trasis, Ans, Belgium

4 University of Liège, Luik, Belgium

Corresponding author: frederik.cleeren@kuleuven.be

Figure S1: Schematic depiction of the cassette and consumables for [18F]AlF-FAPI-74 production on the Trasis AllinOne unit

Table S1: Positioning of reagents/materials on cassettes

| Position | Reagent/Material |
| --- | --- |
| 1, horizontal | Silicone tubing to pressure inlet |
| 1, vertical | Silicone tubing to [18O]H2O recovery vial |
| 2 | QMA eluent |
| 3 | 3 mL syringe |
| 4 | Silicone tubing to QMA cartridge at position 5 |
| 5 | QMA cartridge |
| 6 | 20 mL syringe (without a plunger, activity inlet) |
| 7 | Silicone tubing to reactor |
| 8 | Buffered Al(III) solution |
| 9 | 20 mL syringe |
| 10 | Precursor (in COP vial) |
| 11 | Acetonitrile |
| 12 | Ascorbic acid |
| 13 | tC18 cartridge |
| 14 | Silicone tubing to tC18 cartridge on position 13 |
| 15 | Ethanol |
| 16 | Sodium ascorbate |
| 17 | Silicone tubing to water for injection |
| 18, horizontal | Silicone tubing to exhaust |
| 18, vertical | Alumina N + Silicone tubing to dispensing cell |

Visual representation of the LigandTracer Kinetic Evaluation

Figure S2: Simple visual representation of LigandTracer Kinetic Evaluation set-up, redrawn with a licensed version of Biorender.com [1].

LC/HRMS-analysis

To further identify the radiolabeled compound, its molecular weight was determined using electrospray ionization mass spectrometry (ESI-MS). The corresponding spectra for both the starting material and commercial standard are also provided for comparison.

Figure S3: LC/HRMS-analysis of FAPI-74 precursor

Figure S4: LC/HRMS-analysis of AlF-FAPI-74 standard

Figure S5: LC/HRMS-analysis of [18F]AlF-FAPI-74, with trace amounts of [18F]AlF-FAPI-74 eluting at 11.8 minutes. Mass spectrum at this timepoints shows a main peak for 390.85 m/z value, corresponding with the m/z value of the reference AlF-FAPI-74 compound ([M+2H]2+ = 390.67).

Figure S6: Radio-signal of [18F]AlF-FAPI-74 in LC/HRMS analysis, confirming the retention time of approximately 12 minutes for [18F]AlF-FAPI-74.

Stability assay

Figure S7: Radiochromatograms of [18F]AlF-FAPI-74 stability tests (in formulation vehicle at room temperature), recorded at 2-hour intervals up to 10 hours post end-of-synthesis (EOS), along with the corresponding radiochemical purity (RCP) at each time point.

Radio-metabolite analysis

The in vivo stability of [18F]AlF-FAPI-74 is illustrated through reconstructed radiochromatograms obtained from Gamma Counter output.

Figure S8: Reconstructed chromatogram of radio-metabolite analysis of mouse urine collected 10 min. p.i., showing %intact tracer with mean ± SD (n=3).

Figure S9: Reconstructed chromatogram of radio-metabolite analysis of mouse plasma, showing %intact tracer with mean ± SD (n=3).

Reference List

[1] Bondza S, Foy E, Brooks J, Andersson K, Robinson J, Richalet P, Buijs J. Real-time Characterization of Antibody Binding to Receptors on Living Immune Cells. Front Immunol. 2017 Apr 24;8:455. DOI: 10.3389/fimmu.2017.00455.