Supplementary materials

Table S1 The physicochemical properties of three collected soils

| Depth (m) | Samples | Particle size distribution (%) |  |  | Density (g cm-3) | Porosity | pH | OMC (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Clay (≤ 2 μm) | Silt (2–20 μm) | Sand (20–2000 μm) |  |  |  |  |
| 2~5.5 | Silty loam | 9.3 | 54.9 | 35.8 | 1.40 | 0.46 | 7.26 | 0.66 |
| 0.5~2 | Loam | 4.5 | 31.1 | 64.3 | 1.62 | 0.36 | 8.22 | 1.71 |
| 5.5~6.5 | Sandy loam | 3.3 | 22.2 | 77.8 | 1.45 | 0.44 | 7.34 | 0.22 |

OMC refers to organic matter content.

Table S2 The parameters of the kinetic models to fit the volatilization data on different temperatures

| Soils | Temperature (°C) | k (min-1) | R2 |
| --- | --- | --- | --- |
| Silty loam | 10 | 0.015 | 0.973 |
|  | 15 | 0.016 | 0.980 |
|  | 20 | 0.021 | 0.982 |
|  | 25 | 0.027 | 0.979 |
|  | 30 | 0.037 | 0.990 |
| Loam | 10 | 0.015 | 0.964 |
|  | 15 | 0.019 | 0.984 |
|  | 20 | 0.022 | 0.986 |
|  | 25 | 0.027 | 0.986 |
|  | 30 | 0.037 | 0.987 |
| Sandy loam | 10 | 0.017 | 0.958 |
|  | 15 | 0.023 | 0.990 |
|  | 20 | 0.026 | 0.980 |
|  | 25 | 0.035 | 0.981 |
|  | 30 | 0.049 | 0.989 |

Table S3 The parameters of the kinetic models to fit the volatilization data on different wind speeds

| Soils | Wind speed (m3 min-1) | k (min-1) | R2 |
| --- | --- | --- | --- |
| Silty Loam | 0 | 0.021 | 0.982 |
|  | 2.6 | 0.10 | 0.979 |
|  | 3.1 | 0.10 | 0.973 |
|  | 3.6 | 0.11 | 0.984 |
| Loam | 0 | 0.022 | 0.986 |
|  | 2.6 | 0.13 | 0.990 |
|  | 3.1 | 0.16 | 0.974 |
|  | 3.6 | 0.20 | 0.986 |
| Sandy loam | 0 | 0.026 | 0.980 |
|  | 2.6 | 0.15 | 0.996 |
|  | 3.1 | 0.20 | 0.996 |
|  | 3.6 | 0.22 | 0.992 |

Table S4 The parameters of the kinetic models to fit the volatilization data on different moisture content

| Soils | Moisture content (%) | k (min-1) | R2 |
| --- | --- | --- | --- |
| Silty loam | Air-dried | 0.021 | 0.982 |
|  | 10 | 0.021 | 0.971 |
|  | 20 | 0.023 | 0.975 |
|  | 30 | 0.033 | 0.979 |
| Loam | Air-dried | 0.022 | 0.986 |
|  | 10 | 0.026 | 0.974 |
|  | 20 | 0.027 | 0.975 |
|  | 30 | 0.040 | 0.972 |
| Sandy loam | Air-died | 0.026 | 0.980 |
|  | 10 | 0.032 | 0.978 |
|  | 20 | 0.035 | 0.980 |
|  | 30 | 0.050 | 0.980 |

Table S5 The results of correlation analysis between various influencing factors and the volatilization rate

| Correlation analysis | median particle size | OMC | Temperature | Wind speed | Moisture content |
| --- | --- | --- | --- | --- | --- |
| Zero-order | 0.188 | -0.026 | 0.096 | 0.939** | -0.255 |
| Partial | 0.605 | 0.221 | 0.350 | 0.962** | 0.238 |

**means significance at the 0.01 level, * means significance at the 0.05 level.

Fig. S1 The location (a) and the stratigraphic profile of AB (b) of study area

Fig. S2 The results of (a) K-CV1, (b) K-CV-2, (c) K-CV3 and (d) K-CV4 by BPNN (K-CV refers to the abbreviation of K-folder cross validation)