[Supplementary Information]

Cascaded compressed-sensing single-pixel camera for high-dimensional imaging

Jongchan Park1 and Liang Gao1,*

1Department of Bioengineering, University of California, Los Angeles, CA 90095, USA

*Correspondence: Prof. Liang Gao, Email: gaol@ucla.edu

Fig. S1. Experimental demonstrations of cascaded compressed measurements at varying sampling rate. A butterfly image was captured and reconstructed at varying sapling rate. The reconstructed image is shown in the figure. Root mean squared error (RMSE) and structural similarity (SSIM) of the image are shown in the graph. The pixel resolution of the scene is 384×384. NH and Nθ are numbers of Hadmadard patterns and projection angles, respectively.

Fig. S2. Single-pixel imaging at various DMD refresh rates. a A three-bar object was captured at various DMD refresh rates. The number of Hadamard patterns, projection angles, and spectral bands were 192, 45, and 5, respectively. At a higher DMD frequency (8 kHz), the reconstructed image tends to exhibit more grainy noise. This is attributed to the reduced effective exposure time at the higher DMD refresh rate, where we did not modify the brightness of the object or gain of the single-pixel detector. b Intensity profiles along the white-dashed lines in a.

| Number of spectral bands | Number of Hadamard patterns | Number of rotating angles | Total sampling rate | DMD frequency | DMD picture time | Actual measurement time |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 192 | 45 | 25,920 | 8 kHz | 3.24 s | 3.390 ± 0.032 s |
| 5 | 192 | 45 | 43,200 | 8 kHz | 5.4 s | 5.582 ± 0.017 s |
| 10 | 192 | 45 | 86,400 | 8 kHz | 10.8 s | 11.125 ± 0.014 s |
| 20 | 192 | 45 | 172,800 | 8 kHz | 21.6 s | 22.229 ± 0.027 s |

Table. S1. Measurement time of the cascaded single-pixel imaging system at various numbers of spectral sampling rates. The total measurement time is primarily determined by the picture time of the DMD, rather than the speed of the rotating dove prism. During each measurement, the dove prism rotates 90 degrees, resulting in a 180-degree rotation of the image. The rotary stage (ACR32UT, IntelLiDrivers) used in the experiment has the maximum speed of 1,400 RPM, which is significantly faster than the rotating speed required for the measurement. The actual measurement time was slightly longer than the DMD picture time (of approximately 4%), mainly due to the communication time between the DMD, digitizer, and rotary stage.

Fig. S3. Cascaded compressed sensing for hyperspectral imaging versus random binary encoding for spatially and spectrally multiplexed single-pixel imaging. a Numerical simulations comparing our method to single-pixel compressed multispectral imaging based on random binary pattern encoding1. A standard 3D Shepp-Logan phantom of size (128×128×8) was used as the hyperspectral scene, with varying sampling rates and white random Gaussian noise added to achieve a signal-to-noise ratio of 20 dB. Iterative reconstruction was performed using the FISTA algorithm2 with a 2D total variation regularizer for cascaded compression and a weighted 3D total variation regularizer for random binary encoding. b Reconstructed images at a sampling ratio of 0.05, where Nθ, NH1, Nλ represent the numbers of dove prism angles, 1D Hadamard patterns, and spectral samplings, respectively, while NB2 denotes the number of random binary patterns. c Root-mean-squared error and structural similarity index of the images. The results demonstrate the high efficiency of the cascaded compressed sensing method in sampling the scene at a high sub-Nyquist rate compared to the binary random encoding method. It is worth noting that our method can reconstruct the scene with high fidelity using a simple soft-thresholding operator (MATLAB code in Supplementary Information), while the random binary encoding method requires a computationally heavy total variation regularizer with a heuristic choice of regularization parameters.

Fig. S4. Enlarged view of Fig 5c. In Hadamard compression method with a very sparse sampling, the reconstructed image tends to exhibit pixelation artifact along x and y direction. This occurs because the impinging Hadamard patterns lack high spatial frequency information; we used total variation ordering when choosing the subset of the Hadamard basis because it typically yields a high-quality image at a low sampling rate. On the other hand, the cascaded compression method tends to exhibit smoother boundary and provides higher structural similarity and low root mean squared error given the same sampling rate (Fig. 5).

. a Numerical simulations comparing our method to single-pixel compressed multispectral

Reference

1.	Li, Z. et al. Efficient single-pixel multispectral imaging via non-mechanical spatio-spectral modulation. Sci. Rep. 7, 41435 (2017).

2.	Beck, A. & Teboulle, M. A fast iterative shrinkage-thresholding algorithm for linear inverse problems. SIAM J. Imaging Sci. 2, 183–202 (2009).

MATLAB code for cascaded compressed-sensing single-pixel imaging

% README

% Simulation code for 'Cascaded compressed-sensing single-pixel camera for high-dimensional optical imaging' by Jongchan Park and Liang Gao

% Codes are modified from https://github.com/Computational-Imaging-Hub/LIFT

% Requirements: MATLAB (Tested in MATLAB R2023a), Optional: CUDA-capable GPU device for acceleration

% Instructions: Copy all the codes into a single MATLAB file and run

% In this demo, an image of size (384x384x10) is reconstructed. Expected run time is within 10 seconds.

# Initialization

% 3D image to be measured

imgSize = 384; % image size

lambdaNo = 10; % number of spectrum

phantomImg = repmat(phantom('Modified Shepp-Logan',imgSize),[1, 1, lambdaNo]); % 3D image example

% Sampling

anglesCounter = 45; % number of dove prism angles

hCounter = 192; % number of DMD patterns

snrVal = 20; % signal to noise ratio in dB

# Compressed measurement

% 1. en-face parallel projection

rotAngles = linspace(0,180*(anglesCounter-1)/anglesCounter,anglesCounter); % projection angles

liftImg = zeros(imgSize,anglesCounter,lambdaNo);

for k = 1:lambdaNo

projImg = radon(phantomImg(:,:,k),rotAngles); % radon transform

liftImg(:,:,k) = projImg(round(size(projImg,1)/2-imgSize/2)+1:round(size(projImg,1)/2+imgSize/2),:); % crop

end

% 2. DMD pattern encoding

% Hadamard pattern with a total variation order

hPattern = hadamard(imgSize,'single');

maskTVIndex = zeros(1,imgSize);

for k = 1:imgSize

Gy = gradient(hPattern(:,k));

maskTVIndex(k) = sum(sqrt(Gy.^2)); % 1D total variation

end

[B sortIndex] = sort(maskTVIndex); % sorting the patterns

% Encode the pattern

ySize = round(hCounter); xSize = imgSize;

sensingMatrix = zeros(ySize,xSize);

for k = 1:ySize

sensingMatrix(k,:) = hPattern(:,sortIndex(k))';

end

sensingMatrix = 1/2*(sensingMatrix+1); % DMD display pattern

invSensingMatrix = pinv(sensingMatrix);

totalMeasurement = zeros(ySize,anglesCounter,lambdaNo);

for k = 1:anglesCounter

for l = 1:lambdaNo

totalMeasurement(:,k,l) = sensingMatrix*liftImg(:,k,l);

end

end

% Add noise

totalMeasurement = awgn(totalMeasurement,snrVal);

# Reconstruction

% Parameters for the image reconstruction algorithm

options = [];

options.lambda_reg = 5e-1; % regularization parameter

options.maxIter = 50; % number of iterations

options.useGPU = 1; % GPU usage

options.vis = 0; % display objective

options.positivity = 1; % non-negative intensity

options.rotAngles = rotAngles;

forwardFunction = @(x) forwardComp(x,rotAngles,sensingMatrix,options.useGPU); % forward

backwardFunction = @(x) backwardComp(x,rotAngles,invSensingMatrix,imgSize,lambdaNo,options.useGPU); % backward

options.step = power_iter_cube(forwardFunction,backwardFunction,zeros(imgSize,imgSize,lambdaNo)); % step size

% Image reconstruction

img_recon= fx_rotLIFT(totalMeasurement,options,forwardFunction,backwardFunction,sensingMatrix);

% Visualization

f = figure(2);

subplot(1,2,1); imagesc(phantomImg(:,:,1)); axis('image'); axis off; title('ground truth');

subplot(1,2,2); imagesc(img_recon(:,:,1)); axis('image'); axis off; title('reconstruction');

# Functions

function im_recon = fx_rotLIFT(image_t, options,forwardFunction, backwardFunction,sensingMatrix)

% Initialize parameters

angle = options.rotAngles;

angle = 90 - angle;

N_angle = length(angle);

N_img_comp = size(sensingMatrix,1);

N_img = size(sensingMatrix,2);

Nt = size(image_t,3);

opt.maxiter = options.maxIter;

opt.lambda = options.lambda_reg;

opt.vis = options.vis;

opt.step = options.step;

opt.POScond = options.positivity;

% Setup solver

A = forwardFunction;

AT = backwardFunction;

% Input image

image_meas = zeros(Nt,N_img_comp,N_angle);

for l = 1:Nt

for k = 1:N_angle

image_meas(l,:,k) = image_t(:,k,l);

end

end

image_meas = permute(image_meas,[2,3,1]);

b = reshape(image_meas,[N_img_comp*N_angle*Nt,1]);

b = (single(b)); x0 = zeros(N_img,N_img,Nt);

% Reconstruction

x = Solver_PlugPlay_FISTA(A,AT,b,x0,opt);

im_recon = circshift(flipud(x(:,:,:)),[-1 -1]);

end

% Forward mapping function

function image_F = forwardComp(image_cube,angle,sensingMatrix,useGPU)

N_angle = length(angle);

[m,n,Nt] = size(image_cube);

n_comp = size(sensingMatrix,1);

x = ((1:n)-n/2)/n;

y = x;

[x,y] = meshgrid(x,y);

mask = (x.^2+y.^2)>=0.25;

mask = repmat(mask,[1,1,Nt]);

if(useGPU==1)

line_image_t = single(zeros(N_angle,n,Nt,'gpuArray'));

line_image_t_comp = single(zeros(n_comp,N_angle,Nt,'gpuArray'));

else

line_image_t = single(zeros(N_angle,n,Nt));

line_image_t_comp = single(zeros(n_comp,N_angle,Nt));

end

image_cube(mask) = 0;

for K = 1:N_angle

im = imrotate(image_cube,angle(K),'bilinear', 'crop');

line_image_t(K,:,:) = sum(im,1);

end

line_image_t = permute(line_image_t,[2,1,3]);

for ii = 1:N_angle

for jj = 1:Nt

line_image_t_comp(:,ii,jj) = sensingMatrix*line_image_t(:,ii,jj);

end

end

line_image_t_comp = reshape(line_image_t_comp,[n_comp*N_angle*Nt,1]);

image_F = line_image_t_comp;

end

% Backward mapping function

function img_sum = backwardComp(image_F,angle,invSensingMatrix,n,Nt,useGPU)

if(useGPU==1)

img_sum = single(zeros(n,n,Nt,'gpuArray'));

else

img_sum = single(zeros(n,n,Nt));

end

n_comp = size(invSensingMatrix,2);

N_angle = length(angle);

line_image = image_F(1:n_comp*N_angle*Nt);

line_image = reshape(line_image,[n_comp,N_angle,Nt]);

x = ((1:n)-n/2)/n;     y = x;

[x,y] = meshgrid(x,y);

mask = (x.^2+y.^2)>=0.25;

mask = repmat(mask,[1,1,Nt]);

line = permute(line_image,[2,1,3]);

for k = 1:N_angle

tempLine = invSensingMatrix*squeeze(line(k,:,:));

image_y = permute(reshape(repmat(tempLine,[n,1,1]),[n,n,Nt]),[2 1 3]);

img_sum(:,:,:) = img_sum(:,:,:) + imrotate(image_y, -angle(k),'bilinear', 'crop');

end

img_sum(mask) = 0.;

end

% Optimization step size

function max_egival = power_iter_cube(A,At,image)

max_iter=100;

b0= single(rand(size(image)));

b0=b0./(sum(b0.^2,'all')^(1/2));

iter=0; cond=1;

while(cond)

iter=iter+1;

if(iter==1)

b_old=b0;

end

b_new=At(A(b_old));

b_old_temp=b_old(:);

u_new=b_old_temp.'*b_new(:);

b_new=b_new./(sum(b_new.^2,'all')^(1/2));

if(iter>1)

if((u_new-u_old)/u_new<1e-3)||(iter>=max_iter)

cond=0;

end

end

b_old=b_new;

u_old=u_new;

end

max_egival=u_new;

end

% Solver : Fast Iterative Shrinkage-Thresholding Algorithm

function [x] = Solver_PlugPlay_FISTA(A,At,b,x0,opt)

fprintf(' - Running FISTA with fixed step size method\n'); tic;

lambda = opt.lambda;

maxiter = opt.maxiter;

L0 = opt.step;

POSCOND= opt.POScond;

vis = opt.vis;

xk = x0; yk = xk; tk = 1;

objk  = 0.5*(sum((b - A(xk)).^2,'all')^(1/2));

fprintf('%6s %9s %9s\n','iter','objective','sparsity');

fprintf('%6i %9.2e %9.2e\n',0,objk,nnz(xk)/numel(xk));

for i = 1:maxiter

x_old = xk; y_old = yk; t_old = tk;

yg = y_old - 1/L0*(At(A(y_old)-b));

zk = wthresh(yg,'s',lambda/L0);

xk = zk;

if(POSCOND)

xk(xk<0)=0;

end

tk = (1+sqrt(1+4*t_old*t_old))/2;

yk = xk + (t_old-1)/tk*(xk-x_old);

if(POSCOND)

yk(yk<0)=0;

end

if vis > 0

fprintf('%6i %9.2e %9.2e\n',i,0.5*(sum((b - A(xk)).^2,'all')^(1/2)),nnz(xk)/numel(xk));

end

if (i==1||i==maxiter)

fprintf('%6i %9.2e %9.2e\n',i,0.5*(sum((b - A(xk)).^2,'all')^(1/2)),nnz(xk)/numel(xk));

end

end

x = xk; toc;

end