SUPPLEMENTARY METHODS

Micro Fourier Transform Infrared microscopy

Micro Fourier Transform Infrared microscopy (μ-FTIR) measurements were performed using a Thermo Nicolet iN10 micro Fourier transform infrared microscope. Spectra were recorded in the wavelength range from 4000 to 675 cm-1 using a spectral resolution of 8 cm-1. Spectra were recorded in transmission mode by transferring a small amount of the microplastics onto a diamond compression cell. Transmission analyses were performed to determine and confirm the “average” plastic composition of the microplastics.

Fluorescent microscopy on apical and basolateral InTESTine™ medium

Apical dose solutions of PS-F 0.05, 1 and 10 (1 mg/mL) and InTESTine™ basolateral medium from porcine colon exposed for 5 hours to these dose solutions were prepared for fluorescence microscopy on glass slides with cover slip. Images were collected with a Zeiss Axio Observer Z1 Inverted Microscope. Equipped with a LD condenser 0.35 H/DIC, Ph0, Ph1, Ph2, Varl; 5 positions with aperture diaphragm, WD=70mm. Images are made with objective A-Plan 10x/0.25 Ph1 M27 Air(WD=4.5mm) and objective LD Plan-Neofluar 40x/0.6 Corr Ph2 M27 AIR (D=0-1.5mm) (WD=3.3mm at D=0mm and WD=2.5mm at D=1.5mm. Using the sideport 60N L (100% vis), camera Adapter 60N-C 1’’ 1.0x, Eyepiece PL 10x/23 Br. Foc., Filter Endow GFP shift free refr. 41017 Ex BP 470/40, BS FT 495, EM BP 525/50. Microscopic setting were controlled using the Imaging Software Zen 2 (blue edition).