# Supplementary Material

Status quo of operation procedures for soil sampling to analyze microplastics

Kristof Dorau*1, Martin Hoppe1, Jan Köser1, Georg Scheeder1, Daniel Rückamp1, Katrin Scholz2, Elke Fries1

1 Federal Institute for Geosciences and Natural Resources (BGR), Stilleweg 2, 30655 Hannover, Germany

2 German Environment Agency (UBA), Wörlitzer Platz 1, 06844 Dessau-Roßlau, Germany

*Corresponding author: Kristof.Dorau@bgr.de

Figures: 5

## Figure S1

Articles per year addressing MPs research for the media soil, air, and water (A) and magnification of the development trend over time (B).

## Figure S2

Metadata of research articles that investigate MPs in soil.

## Figure S3

Metrics about procedures regarding sampling time (A), number of sampling depths under investigation per article (B), number of samples investigated per article (C), number of single samples (D), the number of samples merged into composite samples (E), and the final mass of each composite sample (F).

## Figure S4

Metrics for the density separation (DS) procedure and soil organic matter (SOM) removal by application of hydrogen peroxide.

## Figure S5

Most common units employed for the 106 articles within this structured review.

## Table S1

Overview of groups and categories of sampling strategies for MPs research.

| Group | Category | Comment | Unit |
| --- | --- | --- | --- |
| Input pathway | Pathway | Entry of MP into soils, either localized or diffuse. | [-] |
|  | Group | More specialized information about the depositional environment or process. | [-] |
|  | Source | Who is responsible for MP pollution. | [-] |
| Study site | Size | Area of the study site for which the sample is representative. | [m2] |
|  | Soil classification | Description of the soil type/order under investigation. | [-] |
|  | Soil management | Information about management practices (e.g., compost, sewage sludge). | [-] |
|  | Land use | Land use classification (e.g., pasture, arable land). | [-] |
|  | Soil properties | Soil properties that were measured concomitantly with MP contents (e.g., pH, texture). | [-] |
| Sampling design | Sketch | Sketch of the sampling design included (e.g., location and/or distance of the sampling plots). | [-] |
|  | DIN/ISO/CEN | Sampling based on an existing technical note. | [-] |
|  | Sampling time | Season of the year when sampling was conducted. | [-] |
|  | Sampling device | Tool used to excavate the soil sample (e.g., spade, auger). | [-] |
|  | Sampling design | Design for soil sampling (e.g., random, raster, transect). | [-] |
|  | Sampling depth | Depth from which the sample was obtained. | [cm] |
|  | Depths | Number of depths differentiated for the study. | [n] |
|  | Single sample | Single samples per composite sample taken per field. | [n] |
|  | Composite sample | Representative composite samples per field. | [n] |
|  | Composite sample mass | Mass per composite sample. | [kg] |
|  | Total number of samples analyzed | Number of individual samples from which particle counts or mass were derived. | [n] |
|  | Storage | Device used to transport or store the sample (e.g., aluminum box, glass bottle). | [-] |
|  | Homogenization in the field | How the bulk sample was mixed in the field (e.g., by hand, mixing and quartering). | [-] |
| Sample preparation | Homogenization in the laboratory | How the bulk sample was mixed in the laboratory (e.g., mortar, sample splitter). | [-] |
|  | Drying | Drying procedure (e.g., oven drying, freeze drying, air drying). | [-] |
|  | Drying temperature | Drying temperature. | [°C] |
|  | Mesh size | Diameter of the mesh used to physically sieve the soil sample. | [mm] |
|  | Density separation (DS) | Mass of soil taken for density separation and/or soil organic matter removal. | [g] |
|  | DSP solution | Density of the solution used for density separation. | [g/cm3] |
|  | Pretreatment | Extracting agent used to remove soil organic matter (e.g., H2O2, Fenton’s reagent). | [-] |
| Analytical methods | Method | Spectroscopic or thermogravimetric (e.g., ATR-FTIR, SEM, Pyr-GC‒MS). | [-] |
|  | Polymers | Characteristic polymers under investigation. | [-] |
|  | Unit | Unit of the particle count or mass (e.g., P/kg, P/m2). | [-] |
|  | Analytical result | Quantification of polymers as either counts (particles/weight) or contents (mass/weight). | [-] |
|  | Reference material | Application of a recovery test or a reference soil (known MP content). | [-] |
|  | LOQ/LOD and size information | Lowest possible concentration or particle size the method can detect. | [-] |

## Table S2

Summary statistics of groups and categories of sampling strategies for MPs research. For each category, the data were calculated from the raw data (white background) and from processed data by outlier removal (gray background). Outliers were identified by the interquartile range (IQR: 1.5 times higher than the 75th and 1.5 times lower than the 25th percentile of the data). IQR data are discussed in this manuscript.

| Group | Category | Unit | Cases | Mean | Median | SD | Min | Max | 25th perc. | 75th perc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Study site | Size | m2 | 48 | 1032750 | 100 | 4865419 | 0.04 | 32000000 | 5.5 | 3125 |
|  |  |  | 39 | 736 | 25 | 1452 | 0.04 | 5130 | 2.1 | 550 |
| Sampling design | Single samples | n | 74 | 5.1 | 4.5 | 4.6 | 1 | 30 | 3 | 5 |
|  |  |  | 65 | 3.7 | 3.0 | 1.6 | 1 | 7 | 3 | 5 |
|  | Mixed sample | n | 79 | 1.78 | 1 | 1.7 | 1 | 12 | 1 | 2 |
|  |  |  | 72 | 1.36 | 1 | 0.7 | 1 | 3 | 1 | 1 |
|  | Total samples | n | 80 | 79 | 35 | 143 | 2 | 1100 | 16 | 73 |
|  |  |  | 72 | 43 | 30 | 37 | 2 | 150 | 15 | 60 |
|  | Mixed sample mass | g | 73 | 2155 | 1150 | 3674 | 10 | 20000 | 500 | 2000 |
|  |  |  | 68 | 1321 | 1000 | 1073 | 10 | 4000 | 408 | 2000 |
| Sample preparation | Drying temp. | °C | 50 | 53.5 | 55 | 15.3 | 20 | 105 | 40 | 60 |
|  |  |  | 49 | 52.4 | 50 | 13.5 | 20 | 80 | 40 | 60 |
|  | Analytic mass | g | 65 | 125 | 50 | 174 | 0.05 | 1000 | 20 | 200 |
|  |  |  | 60 | 84 | 50 | 87 | 0.05 | 400 | 18 | 150 |
|  | Treatment volume | mL | 26 | 84 | 50 | 75 | 10 | 250 | 30 | 100 |
|  |  |  | 25 | 77 | 50 | 68 | 10 | 200 | 30 | 100 |
|  | Treatment temp. | °C | 32 | 54 | 60 | 16 | 25 | 75 | 50 | 66 |
|  |  |  | 26 | 60 | 60 | 9 | 3 | 75 | 52 | 70 |
|  | Treatment duration | h | 37 | 51 | 48 | 40 | 1 | 192 | 24 | 72 |
|  |  |  | 35 | 44 | 48 | 26 | 1 | 72 | 24 | 72 |
| Particle counts | Aeolian | P/kg | 25 | 16.1 | 0.44 | 68 | 0.00002 | 340 | 0.20 | 0.89 |
|  |  |  | 19 | 0.42 | 0.32 | 0.33 | 0.00002 | 1.1 | 0.17 | 0.63 |
|  | Marine |  | 142 | 1020 | 106 | 2948 | 1.3 | 16867 | 41.5 | 351 |
|  |  |  | 97 | 147 | 84 | 167 | 1.3 | 800 | 40.0 | 179 |
|  | Fluvial |  | 93 | 3868 | 469 | 12260 | 0.37 | 84824 | 90 | 1563 |
|  |  |  | 71 | 420 | 322 | 454 | 0.37 | 2004 | 38 | 633 |
|  | Agriculture |  | 175 | 7927 | 306 | 55354 | 0.23 | 690000 | 38.5 | 2154.5 |
|  |  |  | 139 | 854 | 231 | 1199 | 0.23 | 6235 | 25.0 | 1322.0 |
|  | Urban |  | 71 | 11444 | 4518 | 27830 | 100 | 218182 | 1150 | 11882 |
|  |  |  | 61 | 6176 | 3877 | 6481 | 100 | 26761 | 1100 | 10572 |