# Supplementary Information

Related manuscript: Microplastics in a Highway Stormwater System: Concentrations and Variability During Rainfall

# Authors

Elly Lucia Gagginia, Ann-Margret Strömvalla, Elisabeth Støhle Rødlandb, Ekaterina Sokolovac, Yvonne Andersson-Sköldd,e, Mia Bondelinda*

a Water Environment Technology, Department of Architecture and Civil Engineering, Chalmers University of Technology, SE-412 96 Gothenburg, Sweden

b Norwegian Institute for Water Research, Økernveien 94, NO-0579 Oslo, Norway

c Program for Air, Water and Landscape Sciences; Hydrology, Department of Earth Sciences, Uppsala University, SE-752 36 Uppsala, Sweden.

d Swedish National Road and Transport Research Institute (VTI), Box 8072, SE-402 78 Gothenburg, Sweden

e Division of Geology and Geotechnics, Department of Architecture and Civil Engineering, Chalmers University of Technology, SE-412 96 Gothenburg, Sweden

*Corresponding author: Mia.Bondelind@chalmers.se, Department of Architecture and Civil Engineering, Chalmers University of Technology, SE-412 96 Gothenburg, Sweden

# Additional information about analytical methods

Table SI 1: List of marker compounds used for quantification of microplastic polymers by commercial laboratory.

| Polymer type | Marker compound | Marker ion (m/z) |
| --- | --- | --- |
| ABS | 2-Phenethyl-4-phenylpent-4-enenitrile | 170 |
| PE | 1,20-Heneicosadiene | 82 |
| PP | 2,4-Dimethyl-1-heptene | 126 |
| PS | 2,4,6-Triphenyl-1-hexene | 91 |
| SBR | 4-Phenylcyclohexene | 104 |
| PMMA | Methyl methacrylate | 100 |
| PC | Bisphenol A | 213 |
| PVC | Napthalene | 128 |
| PET | Benzoic acid | 122 |
| N6 | Caprolactam | 113 |
| N66 | Cyclopentanone | 84 |
| NR | Dipentene | 93 |
| BR | 4-vinylcyclohexene | 79 |

The compound 4-vinylcyclohexene (4-VCH) is not specific for BR but also receives contributions from butadiene-monomers in ABS and SBR, and some additional contribution from m/z 79 from 2,4-dimethyl-1-heptene for PP with approx. same retention time as 4-vinylcyclohexene. The contribution from these polymers is calculated and subtracted during data post-processing. Ratio of contribution is recalculated for each recalibration. Hence, reported BR is not the total mass of butadiene components found, as the contribution from polymers SBR and ABS is subtracted.

# Information on the samples aggregated for MP analysis

Figure SI 1: Image showing how bottles sampled on 2023-04-29 were aggregated for the MP analysis, as listed in Table 1. Black circles indicate which bottles were aggregated and analysed for MP.

Figure SI 2: Image showing how bottles sampled on 2023-05-17 were aggregated for the MP analysis, as listed in Table 1, with zoomed in views at certain times marked by brown (ca. 06:30 to 08:00), purple (ca. 14:30 to 15:00) and green (18:30 to 18:40) boxes for better visualization. Black circles and arrows indicate which bottles were aggregated and analysed for MP.

# Tabulated measured MP concentrations in gully pots and wells of Testsite E18

Table SI 2: Microplastic polymer concentrations for the selected aggregated samples from the rain events on 2023-04-29 and 2023-05-17.

| Name | Aggregated time samples | PE [µg/L] | PP [µg/L] | PS [µg/L] | ABS [µg/L] | PMMA [µg/L] | PC [µg/L] | PVC [µg/L] | PET [µg/L] | PA6 [µg/L] | PA 66 [µg/L] | PB [µg/L] | PI [µg/L] | SBR [µg/L] | SBR+BR [µg/L]a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP1b 2023-04-29 | 17:02 17:04 | 1400 | <0.50 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | 1000 | 1800 | 29 000 |
| GP1 2023-04-29 | 17:12 17:14 | <0.10 | 11 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | 3.8 | <0.20 | 7400 |
| GP2c 2023-04-29 | 16:59 17:02 | <0.10 | 49 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | 3.6 | 2.8 | <0.20 | 54 000 |
| GP2 2023-04-29 | 17:04 17:07 | <0.10 | 4.6 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 38 000 |
| GP2 2023-04-29 | 17:09 17:12 | 170 | <0.5 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | 340 | 21 000 |
| GP2 2023-04-29 | 17:19 17:22 | <0.10 | 11 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | 1.3 | <0.20 | 7000 |
| GP2 2023-04-29 | 17:29 17:32 | <0.10 | 13 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | 1.7 | 5300 |
| GP2 2023-04-29 | 17:52 17:54 | <0.10 | 5.0 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | 0.20 | <3.0 | <1.0 | <1.0 | <0.20 | 3200 |
| WAd 2023-04-29 | 17:02 17:04 | <0.10 | 11 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 35 000 |
| WA 2023-04-29 | 17:12 17:19 | <0.10 | 12 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 26 000 |
| GP1 2023-05-17 | 18:14c 18:16 c | <0.10 | 5.4 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | 7.0 | <0.20 | 24 000 |
| GP1 2023-05-17 | 18:21 c 18:29 | <0.10 | 13 | <1.0 | <0.20 | <0.20 | <1.0 | 17 | <0.30 | <0.10 | <3.0 | 2.0 | 4.1 | 8.7 | 10 000 |
| GP2 2023-05-17 | 06:54 06:57 | 190 | <0.50 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | 270 | 8800 |
| GP2 2023-05-17 | 06:59 07:04 | <0.10 | 55 | <1.0 | <0.20 | <0.20 | <1.0 | 3.2 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 12 000 |
| GP2 2023-05-17 | 07:12 07:19 | <0.10 | 21 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 10 000 |
| GP2 2023-05-17 | 07:57c 08:27c | <0.10 | 45 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 8200 |
| GP2 2023-05-17 | 14:44c 14:47c 14:49c | 5.8 | <0.50 | <1.0 | 1.2 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | 5.9 | 10 | 3400 |
| WA 2023-05-17 | 18:14c 18:17c | <0.10 | 4.3 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | 1.6 | 1.0 | <0.20 | 22 000 |
| WA 2023-05-17 | 18:24c 18:37 | <0.10 | 2.4 | <1.0 | <0.20 | <0.20 | <1.0 | <3.0 | <0.30 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 | 18 000 |
| Blank |  | <0.10 | 1.4 | <1.0 | 1.4 | <0.20 | <1.0 | <3.0 | 0.40 | <0.10 | <3.0 | <1.0 | <1.0 | <0.20 |  |
| LoQ |  | 0.10 | 0.50 | 1.0 | 0.20 | 0.20 | 1.0 | 3.0 | 0.30 | 0.10 | 3.0 | 1.0 | 1.0 | 0.20 |  |
| a indicates calculated SBR+BR concentrations for the corresponding aggregated samples of the present study based on the results of Gaggini et al. (2025); b GP1 = sample collected in gully pot 1, see section 2.1; c GP1 = sample collected in gully pot 1, see section 2.1; d WA = sample collected in gully pot 1, see section 2.1. c Samples analysed for SBR+BR with marker combination excluding benzene (m/z 118 Da α-methylstyrene, m/z 117 Da ethylstyrene and m/z 91 Da butadiene trimer) in Gaggini et al. (2025) due to high level of benzene, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Metal concentrations on aggregated samples

Figure SI 3: For location GP2, metal concentrations from rain on 2023-04-29. Calculated concentrations of total metals for the aggregated samples based on the results by Gaggini et al. (2025), with different y-axis between top and bottom figure.

# References

Gaggini EL, Sokolova E, Rødland ES, Strömvall AM, Andersson-Sköld Y, Bondelind M. Tyre wear particles in a highway stormwater system during rain: quantification by automatic sampling and pyrolysis-GC/MS, and correlations with metals and solids. Environ Sci (Camb) 2025;11:3001–13. https://doi.org/10.1039/d5ew00656b.