Appendix A

Table A1. System Operating Parameters.

| Unit | Parameter | Value |
| --- | --- | --- |
| MGT | PMT,max (kW) | 300 |
|  | ζMT | 0.30 |
|  | Qgas (kWh /m3) | 10 |
|  | cgas (CNY/m3) | 2.5 |
|  | kMT (CNY/kWh) | 0.20 |
| Biomass Generation Unit | Pbio,max (kW) | 500 |
|  | Qmat (kcal/kg) | 3200 |
|  | cmat (CNY/t) | 300 |
|  | kbio (CNY/kWh) | 0.45 |
|  | ηbio | 0.65 |
| Grid Interaction | Pgrid,maxbuy (kW) | 1500 |
|  | Pgrid,maxsell (kW) | 500 |
|  | ηAC-DC、ηDC-AC | 0.95 |
| ESS | EsN (kWh) | 3000 |
|  | Pes,maxcha (kW) | 800 |
|  | Pes,maxdis (kW) | 800 |
|  | ηescha、ηesdis | 0.90 |
|  | kes (CNY/kWh) | 0.05 |
| DR Load | Pexe,max (kW) | 300 |
|  | Pcut,max (kW) | 300 |
|  | ku (CNY/kWh) | 0.30 |
| EV | Ebat,n (kWh) | 60 |
|  | Prate,ncha (kW) | 7 |
|  | Prate,ndis (kW) | 3.3 |
|  | SOCmax,n | 0.80 |
|  | SOCmin,n | 0.20 |
|  | N | 100 |
|  | ηevcha、ηevdis | 0.95 |
|  | Ncyc,ref (cycles) | 5000 |
|  | cbatunit (CNY/kWh) | 500 |
| Market Trading | λc (CNY/t) | 70 |
|  | λccer (CNY/t) | 100 |
|  | λgec (CNY/MWh) | 50 |
|  | βccer | 0.50 |

Table A2. TOU Electricity Price Parameters.

|  | Time Period | Purchase Price (CNY/kWh) | Selling Price (CNY/kWh) |
| --- | --- | --- | --- |
| Valley Period | 00:00-10:00 | 0.29 | 0.20 |
| Flat Period | 12:00-14:00, 19:00-24:00 | 0.71 | 0.50 |
| Peak Period | 10:00-11:00, 14:00-15:00, 17:00-19:00 | 1.19 | 0.83 |
| Critical Peak Period | 11:00-12:00, 15:00-17:00 | 1.48 | 1.00 |

Appendix B

Pareto Analysis of the Multi-objective Optimization Results

To analyze the trade-off between the two optimization objectives, the ε-constraint method is employed to obtain representative Pareto solutions. The load regulation objective FLis imposed as a constraint, while the optimization objective cost Fcostis minimized within the feasible region of the original dispatch model:

|  | (B1) |
| --- | --- |

where Ωdenotes the feasible region of the original dispatch model, and εirepresents different load-regulation constraint levels. Twenty-one εlevels are uniformly selected between FLminand FLrefdefined in Section 4.2.3 to obtain representative Pareto solutions.

| (a) Pareto front in the objective space | (b) Economic evaluation of Pareto solutions |
| --- | --- |

Figure B1. Multi-objective trade-off analysis

As shown in Fig. B1(a), the optimization objective cost Fcostgradually decreases as the allowable load peak-to-valley difference increases, indicating a clear trade-off between the two optimization objectives. Fig. B1(b) further evaluates the same scheduling solutions using the system net operating cost Csysadopted in the result statistics. It should be noted that Fcostand Csyshave different cost-accounting scopes. In particular, the CERF-based carbon-responsibility guidance cost is introduced as an internal low-carbon guidance term in the optimization objective and is excluded from the system net operating cost used for result statistics. Therefore, Fig. B1(b) presents the corresponding economic performance of the Pareto schedules rather than an additional Pareto front.