Quantitative chemical profiling of commercial glyceride excipients via 1H NMR spectroscopy

Isha Saraf1, Varun Kushwah1, Hansjoerg Weber2, Dattatray Modhave1, Thean Yeoh3, and Amrit Paudel1, 4*

1Research Center Pharmaceutical Engineering (RCPE) GmbH, Inffeldgasse 13, 8010 Graz, Austria

2Institute of Organic Chemistry, Graz University of Technology, Stremayrgasse 9A, 8010 Graz

Austria

3Drug Product Design, Pharmatherapeutics Pharmaceutical Sciences, Worldwide Research and Development, Pfizer Inc, Groton, CT, USA

4Institute for Process and Particle Engineering, Graz University of Technology, Inffeldgasse 13, 8010 Graz, Austria

*Corresponding author:

E-mail: amrit.paudel@tugraz.at

Tel.: +43 316 873 30912

Supplementary Information

T1 relaxation experiment:

T1-Relaxation experiments were performed using the standard mixture (glycerol, mono-, di-glycerides and their isomers, tri-glycerides and fatty acids) dissolved in 0.6 mL of CDCl3 containing 1.0 mg of reference standard (tCNB) as per the protocol mentioned in the section 3.1.

T1-Relaxation experiments have been carried out using the inversion-recovery sequence (d1 - 180° pulse - d2 - 90° pulse - acq) and the following parameters: d1 20 s, number of scans 4, d2 as an array of 10 values from 0.0625 - 32 s.

Result and Discussion

The T1 relaxation values obtained were less than 3 secs for all the signals (Figure S1, S2 and S3). Whereas, for the selected signals used for the quantification of chemical composition (equation 2 to 8) of MDGs, the T1 relaxation values were even lower than 2 sec, suggesting application of sufficient relaxation times for the proton (relaxation delay 1.0 s, number of scans 32, acquisition time 2.049 s).

Figure S1: T1 Relaxation experiments using the inversion-recovery sequence

Figure S2: Relaxation curves of samples (8 most intense peak from the Figure S1)

Figure S3: T1 relaxation time value (of 15 most intense signals from Figure S1)