SUPPLEMENTAL MATERIAL

Viscoelastic and phototunable GelMA-alginate hydrogels for 3D cell culture

Gulistan Tansika and Ryan Stowers*a,b

aDepartment of Mechanical Engineering, University of California, Santa Barbara

bDepartment of Bioengineering, University of California, Santa Barbara

*Correspondence to rstowers@ucsb.edu

GelMA synthesis and characterization

Briefly, porcine skin gelatin (Sigma Aldrich) was dissolved at 10% (w/v) in phosphate buffered saline (PBS) (pH = 7.4) and mixed with MA at a concentration of 0.8 g of MA per 1 g of dissolved gelatin. The mixture was then dialyzed against distilled water (12–14 kDa cutoff dialysis tubing) for five days at 40ºC to remove any unreacted MA groups from the solution. Freeze-dried GelMA was kept at -20ºC until further use. The fluorescamine protein assay was performed to determine the degree of substitution of free amine groups via methacrylation.18 Briefly, a series of dilutions of gelatin ranging from 0 to 2 mg/ml were made using PBS and GelMA was diluted to 1.25 mg/mL in PBS. After that, 90 μL aliquots of samples and standards were pipetted into a 96 well plate with 30 μL of a 3 mg/mL fluorescamine solution prepared in DMSO (n=6). Then, the microplate was placed on a microplate shaker for 30 min at room temperature under shielding from light, and the fluorescence intensity was measured using a plate reader at an excitation wavelength of 360 nm and emission wavelength of 465 nm (BioTek Synergy H1).

Interpenetrating network hydrogel formation

A 10% (w/v) GelMA solution was prepared by dissolving GelMA in DMEM containing 1% (w/v) lithium phenyl-2,4,6-trimethylbenzoylphosphinate (LAP) photoinitiator. Next, alginate (6%, w/v) in DMEM and 1.2 M CaSO4 were prepared. Components were mixed to obtain hydrogels with a final concentration of 2% w/v alginate, 0.5% w/v photoinitiator, 5% w/v GelMA, and 25 mM CaSO4. Then, 900 µl of GelMA-alginate solution was cast between two glass plates and exposed to light (405 nm) with an intensity of 6 mW/cm2 for 5 seconds to crosslink the GelMA into a disc-shaped sample with a diameter of 5 mm and a thickness of 2 mm. Finally, the samples were immersed in DMEM at 37°C for 24 hours to hydrate and equilibrate before mechanical testing. To dynamically stiffen the hydrogels, a sequential crosslinking strategy was used. Briefly, the initially soft hydrogels prepared on day 1 were stiffened by radical polymerization of the remaining methacrylate groups after the hydrogel was swollen with LAP for 30 min and exposed to light for either 15 or 60 seconds, on day 2.

Cell Viability

We assessed the cell viability after 24 h of dynamic stiffening at days 1 and 2 (n=3) using a LIVE/DEAD Cell Imaging Kit (488/570, Molecular Probes, Life Technologies Corp., CA, USA) following the protocol provided by the manufacturer. The kit labels live cells with calcein AM and dead cells with ethidium homodimer. To establish TCP positive control group, D1 cells were seeded at a density of 5×10³ cells/cm² in a 24-well tissue culture plate and cultured under standard conditions without exposure to light radiation. We used a laser scanning confocal microscope (Leica TCS-SP8) to visualize cells. Fluorescence images of live and dead cells were taken from the five different areas of one sample derived from one well at 10× magnification. All samples were analyzed in triplicate. Live and dead cells were counted by using Image J automated cell counting program by first performing a 16-bit grayscale conversion, followed by thresholding and particle analysis. The viability was defined as the number of live cells among the total number of cells.